- Predictive Maintenance System Using Edge-AI Noise Analysis

- Introduction to EV and Predictive Maintenance

- 1.1 Reliability of EV powertrain

- The rapid adoption of electric vehicles (EVs) has shifted automotive powertrain architectures from Internal combustion engines (ICE) to highly integrated electronic systems. Unlike traditional internal combustion engines , the motors in modern EVs operate at exceptionally high speeds to achieve high power density and efficiency. The motors are connected with the controller and inverter which manages the flow of AC current to the stators.

- The powertrain of an EV depends on the vehicle subsystems like motor, Power electronics controller, traction inverter and battery pack. The Reliability of powertrain in EV is important than those of Engine oriented vehicles because even a small failure in a subsystem of a powertrain in EV can cause an abrupt mechanical breakdown which results in instantaneous high speed wheel lock-up. Therefore, ensuring the reliability of these subsystems is a primary focus for proper functioning of the EV.

- 1.2 Predictive maintenance and condition-Based monitoring (CBM)

- To maintain the reliability of modern EV powertrain, the Industry is shifting towards predicitve maintenance and condition based monitoring from the traditional maintenance strategies which is based on preventive frameworks. The most traditional and simplest maintenance strategy is Run-to-failure approach where the maintenance is done after the failure of the subsystems or breakdown. This also includes preventive maintenance where the subsystems are maintained or repaired at periodic intervals, but it is less efficient and cost of failure is high.

- To overcome the limitations of these traditional and conventional strategies, modern EV industry is focused on predicitve maintenance and condition based monitoring (CBM). Predictive maintenance uses real time data to predict the system breakdown. It helps to increase predictive accuracy. This approach helps to fix the issue at the initial stage instead of waiting till the complete breakdown. With the advancements in AI and sensors, real time data processing has become more efficient. Condition based monitoring involves monitoring components in real-time. With the advancement in sensors there is an increase in availability of data like vibration, temperature, pressure, and other kinds of electrical and mechanical equipment condition-monitoring data.

- 1.3 The Role of Acoustics in Early-Stage Degradation Detection

- There are many parameters available for condition based monitoring. One such parameter is vibration of the motors. To implement Condition-Based Monitoring (CBM), the diagnostic system must capture the earliest possible indicators of physical or electrical damage. Traditional vibration sensors are widely used in the automotive industry; however, they possess a critical limitation, they typically only identify abnormalities after macroscopic structural damage such as significant bearing rupture or rotor imbalance has already occurred . By this stage, the EV motor is already compromised.

- To achieve the early stage detection, the vibration based condition monitoring or predictive maintenance system can be used. The systems uses acoustic emission analysis and high frequency acoustic monitoring. With the advancements in signal processing and AI raw vibration can be converted to specific domain (time to frequency) which can further be identified using Edge AI technology. This approach is used to predict and detect the microscopic damage caused in the motors of EV. In the Edge AI predictive maintenance system, a digital microphone is used to continuously sound waves and send this to a trained ML model in frequency domain spectograms which can identify the damage in the early stage and immediate repair can be done.


2. Overview of EV Traction Motors: Architectures and Mechanics

2.1 Classification and Comparative Analysis of EV Motors

There are different types of motor used in automotive industry. The five main electric motor types are DC, induction, permanent magnet synchronous, switched reluctance and brushless DC motors. The selection of a motor for an electric vehicle (EV) involves a critical balance of power density, efficiency, and manufacturing cost. Some types motors used in EVs are permanent magnet synchronous motors (PMSM), Induction motor, Brushless DC motor (BLDC) and synchronous reluctance motors (SynRM).

 Currently PMSMs dominate the EV market due to it’s high efficiency and torque. However, they rely on high usage of rare earth magnetic materials (like neodymium) which leads to high production cost. Induction motors is used as an alternative where the need for permanent magnets is eliminated. Induction motors are cost effective, but provides lower efficiency. BLDC motors can also be used which provide high torque but similar to induction motors, BLDC also exhibits lower efficiency. Recently, the industry have shifted towards Synchronous Reluctance motors (SynRM) and it’s permanent magnet variants (PMa-SynRM). SynRM works on the principle of least resistance path (reluctance). SynRM are highly efficient and cost of production is less. PMa-SynRM which uses permanent magnet on rotor is introduced to provide greater torque than standard SynRM.

2.2 Motor Architecture & Electro-Mechanical Working Principles

The two main components of a motor are: stator and rotor. The fundamental electro-mechanical conversion in EV motors relies on the interaction between the stator and the rotor. The stator consists of a stationary iron core wound with multi-phase copper coils or a permanent magnet based on the type of motor (Induction motor and PMSM). A simple motor consists of armature, commutator and brushes which act as contact between commutator for the rotation.

When  alternating current (AC) is provide through inverter from battery pack, the windings in the stator generates a Rotating Magnetic Field (RMF) which causes the rotor to work and produce mechanical energy for the drive. The stator acts as a electromagnet when the current is passed through the windings. The poles of magnetic field created by stator attracts or repels the poles of the rotor, which leads to the continuous spinning of rotor.

The DC motor has stator and armature wound on the rotor, it uses a simple torque speed characteristics for EV drive. However due to the presence of commutator and brushes the maintenance cost is high. It also has poor regenerative braking capacity. Indction motors use alternating current windings on stator. The RMF induces a voltage across the rotor's copper or aluminum bars, creating an opposing magnetic field that drags the rotor along asynchronously (squirrel cage induction motor). In a PMSM, torque is generated as the rotor's permanent magnets synchronize and align with this RMF for the drive. SynRM uses a rotor that operates on basis of reluctance torque (path of least resistance as the working principle). Since it is synchronous, no EMF is induced in rotor. An advancement to SynRM is PMa-SynRM and it is currently in high demand in EV market due to it higher efficiency.


2.3 Fault Mechanisms and Degraded Acoustic Profiles

When EV motors experience thermal or electrical stress they develop specific fault mechanisms that emit distinct acoustic profiles. The most common mechanical failure is bearing degradation caused by shaft voltages. Each component in the powertrain causes different acoustic emission or profile when damaged. 

High-frequency voltages from the inverter can break down the bearing's lubrication film, causing currents to arc across the metal . This creates microscopic damages, resulting in a low-frequency, rhythmic acoustic clicking or rumbling that scales with the motor's RPM. Excessive thermal stress can melt the stator winding insulation, leading to Short Circuits. This disrupts the balanced phase currents and alters the magnetic pull, creating abnormal acoustic emission. In PMSMs, extreme overheating can cause irreversible demagnetization of the rotor magnets. This magnetic damage  reduces motor efficiency but dramatically increases high-frequency acoustic noise and vibration (electrical whine).

- 3. Edge-AI & TinyML Sound Analysis Pipeline

- 3.1 Acoustic Sensing Technologies

- Sound signals not only serve as the primary communication medium but also find application in fields such as  fault detection.The first critical stage is capturing  acoustic data in environments with severe electromagnetic interference (EMI), such as inside an EV  inverter and motor. Traditional analog microphones require external Analog-to-Digital Converters (ADCs) and are highly susceptible to electrical noise from high-voltage switching cables.

- To solve this, recent embedded frameworks utilizes digital microphone I2S (Inter-IC Sound) interface, such as the INMP441 . By transmitting data digitally via I2S, high-precision audio is sent directly into microcontrollers like the ESP32 without requiring complex analog conditioning circuits like ADCs and sensors. Acoustic sensing is used as an application for different subsystems in EV powertrain. The traditional training technique used for the edge AI approach is to rely  on textual manuals, hands-on demonstrations, and limited audio samples, which restrict learners ability to recognize subtle acoustic signatures indicative of powertrain faults. Deep learning driven acoustic sensing technology uses convolutional neural networks (CNNs) and recurrent neural networks (RNNs) to analyze real-time acoustic signals from electric motors, inverters, and drivetrain components.

- 3.2 Digital Signal Processing (DSP) and Feature Extraction

- Once digital audio is captured, it must be processed. Feeding raw time-domain audio data directly into a model is computationally inefficient for resource constrained microcontrollers. Therefore, Edge-AI pipelines utilize mathematical transformations to extract meaningful features. A standard approach involves applying a Fast Fourier Transform (FFT) over sliding time windows to convert the time-domain signal into a  frequency-domain Spectrogram. This effectively maps sound as an image, making high-frequency anomalies visually distinct.

- Furthermore, advanced techniques utilize Mel-Frequency Cepstral Coefficients (MFCC) or Log-Mel filterbanks. These algorithms compress acoustic features by applying a non-linear frequency scale, which is highly effective at isolating mechanical machine frequencies and filtering out low-frequency background noise. Extracting Mel Frequency Cepstral Coefficients from sound signals and training a lightweight neural network classifier optimized for deployment on edge devices.

- 3.3 TinyML Neural Network Implementations

- The final stage is classifying the processed spectrogram on the edge device. Tiny Machine Learning enables real-time, energy efficient data processing directly on microcontrollers, making it ideal for Internet of Things sensor networks and allows complex AI models to run on ultra-low-power devices. The Digital Signal Processing stage converts the sound into a image (the spectrogram) for the analysis to be efficient.

- A Neural Network is a specific type of mathematical algorithm inspired by the human brain. It is the actual code that looks at data spectrograms, finds patterns, and makes decisions. TinyML (Tiny Machine Learning) is simply the industry term for taking any  Neural Network and compressing it so it can survive on a tiny, low-power chip like an ESP32. For the deployment of detected noise on microcontroller like ESP32, the neural networks are trained with the processed spectogram and quantization is done to provide a compressed lightweight neural network which is known as the TinyML model.

- Patent Landscape & Industrial Standards

4.1 Industrial Standards for Machine Evaluation

- The evaluation of machinery health is governed by the ISO 10816 standard (recently updated and absorbed into the ISO 20816 series). This standard  establishes general conditions and procedures for the measurement and evaluation of vibration using measurements made on rotating, non-rotating and non-reciprocating parts of complete machines.  The standard categorizes machine health into four distinct severity zones: Zone A (proper condition), Zone B (satisfactory unrestricted operation), Zone C (restricted operation/warning), and Zone D (unacceptable/damage occurring). The general evaluation criteria, which are presented in terms of both vibration magnitude and change of vibration, relate to both operational monitoring and acceptance testing.

However, a critical limitation of the ISO 10816 framework is its reliance on macroscopic mechanical shaking. By the time a motor reaches Zone C, physical damage (such as advanced bearing spalling) is already well underway. Consequently, recent Edge-AI acoustic models are designed to complement these standards by capturing ultrasonic and high-frequency acoustic emissions (above 10 kHz) that precede the low-frequency vibrations regulated by ISO 10816. By detecting these early acoustic shifts, intelligent systems can trigger maintenance alerts before the motor formally degrades into an ISO "Zone C" state.

4.2 Patent Ecosystem and Commercial Embodiments

The integration of acoustic sensors and Edge-AI into automotive powertrains is a rapidly expanding area of commercial intellectual property. Major automotive suppliers and manufacturers have filed numerous patents detailing the use of embedded microphones for diagnostic purposes.

Modern patent architectures outline the use of localized microcontroller units (MCUs) to perform frequency-domain transformations (such as Fast Fourier Transforms) directly at the sensor node, transmitting only the final "health state" classification to the vehicle's central control unit rather than streaming raw audio . This localized Edge-AI approach uses ESP32 and I2S microphone methodology in this project which protects driver privacy, drastically reduces data bandwidth, and overcomes the high latency associated with cloud-based systems.

4.3 Commercial Intellectual Property and Structural Localization

The automotive industry is aggressively pursuing machine-learning-based acoustic diagnostics, as evidenced by recent patent filings from major original equipment manufacturers . A prominent example is U.S. Patent US20190257794A1, assigned to General Motors, which outlines a framework for the real-time detection and localization of damage in vehicle components using acoustic emission (AE) and machine learning .

The GM patent details a methodology where a distributed array of piezoelectric AE sensors is physically coupled to the vehicle's structural components. When a structural anomaly (such as a micro-crack or bearing rupture) occurs, the sensors capture the transient elastic stress waves propagating through the solid material. A centralized machine learning algorithm processes the time of arrival differences across the sensor array to precisely map the spatial coordinates (localization) of the damage.

4.4 Cabin-Acoustic Diagnostics and Component-Level Source Separation

The optimization of acoustic monitoring in electric vehicle architectures is further demonstrated in recent patent developments by automotive manufacturers targeting Power Electric (PE) subsystems. Specifically, South Korean Patent Application KR1020230121249A, assigned to Hyundai Motor Company and Kia Corporation, discloses an AI-based noise diagnostic apparatus and method for EV PE components including traction motors, reducers, and inverters.

The Hyundai/Kia architecture implements a multi-channel microphone array arranged within the vehicle cabin to gather multi-directional acoustic profiles spanning the front engine bay, rear trunk, and upward reflection planes. To overcome the challenge of ambient road and wind noise, the framework introduces deep-learning-based source separation models like TCN-based Conv-TasNet and Transformer-based SepFormer architectures to isolate high-frequency PE (power electronics) component "whine noise" from raw time-domain signals. Furthermore, spatial tracking is executed via Generalized Cross Correlation with Phase Transform (GCC-PHAT) to analyze arrival time delays, mapping the fault origin and identifying noise-leak vectors which are subsequently mapped over a camera interface.

5.Comparative Integration with the Proposed System

When evaluating the commercial patent landscape, a distinct trend emerges: state-of-the-art automotive diagnostic architectures rely heavily on multi-sensor, centralized frameworks designed for spatial tracking and localization. The General Motors framework (US20190257794A1) focuses on structure-borne elastic waves captured by a distributed array of specialized piezoelectric sensors to triangulate physical material cracks across the vehicle chassis. Conversely, the Hyundai/Kia architecture detailed in the (KR20230121249A) processes airborne acoustics using an array of interior cabin microphones, leveraging advanced deep-learning source separation models (such as Conv-TasNet and SepFormer) to isolate high-frequency Power Electric (PE) whine noises from environmental driving interference. Both commercial systems utilize multi-channel time-delay calculations such as Generalized Cross Correlation with Phase Transform (GCC-PHAT) in the Hyundai/Kia configuration to map and visualize the exact spatial origin of structural anomalies or acoustic leaks.

While these centralized multi-sensor approaches offer comprehensive vehicle-wide diagnostic capabilities, they introduce extreme algorithmic complexity, high manufacturing costs. The Edge-AI predictive maintenance architecture proposed in this project introduces a highly valuable, decentralized alternative through structural localization. By placing a single, low-cost digital I2S MEMS microphone directly at the specific target asset the motor , the system bypasses the need for multi-channel spatial triangulation and heavy, resource-intensive source separation networks. Rather than streaming raw audio to a high-power centralized processor, baseline digital signal processing (DSP) filters out environmental noise profiles right at the sensing node. This enables a highly compressed, 8-bit quantized TinyML neural network to execute rapid, real-time inference locally inside an ultra-low-cost ESP32 microcontroller. The proposed localized project significantly minimizes latency, slashes computational overhead, and eliminates raw data transmission, offering a highly efficient and a feasible solution for targeted component-level predictive maintenance.

- 6. Summary of Research Gaps and Technical Challenges

6.1 High Computational Complexity vs. Edge Resource Constraints

A prevailing limitation in the current literature is the reliance on high-power computing architectures. A significant portion of acoustic anomaly detection models utilize deep, multi-layered Neural Networks or Transformer-based architectures to process complex audio features like spectrograms or MFCCs . While these models achieve classification accuracies exceeding 95%, they are typically deployed on industrial PCs, high-end processing units, or high power consumong Edge-AI platforms. There is a distinct scarcity of literature optimizing these neural networks to execute continuous, real-time audio inference within ultra-low-power, resource-constrained microcontrollers (MCUs) such as the ESP32. Consequently, research into extremely lightweight, post-training quantized TinyML models remains a critical open gap.

6.2 Vulnerability to Dynamic Ambient Factory and Road Noise

A secondary research gap involves environmental noise robustness. The majority of published studies train and validate their acoustic models using clean datasets recorded in soundproofed anechoic chambers or controlled benches with steady baseline conditions. In real-world applications such as an active manufacturing floor or an operating EV cabin the target motor's high-frequency acoustic signatures are frequently masked by loud, non-stationary background interferences (e.g., wind resistance, tire-road contact, adjacent heavy machinery). Current state-of-the-art commercial systems attempt to solve this via massive, multi-channel microphone arrays and complex source-separation algorithms (e.g., Conv-TasNet) . However, there is a clear gap in developing single-channel, low-overhead digital signal processing (DSP) workflows that can isolate machine "whine" or bearing "clicks" from dynamic background noise directly on low-cost edge nodes.

6.3 Scarcity and Reliance on Supervised Run-to-Failure Datasets

Finally, existing predictive maintenance models heavily depend on supervised learning paradigms, which require extensive, precisely labeled data for both "healthy" and "faulty" operating states. In practical automotive and industrial settings, collecting comprehensive training data for catastrophic structural breakdowns is highly impractical, dangerous, and economically prohibitive. Operators cannot purposefully run expensive EV drivetrains to the point of structural destruction simply to gather training samples. While patents and high-level academic frameworks have proposed complex spatial triangulation models to identify damage as it occurs, there is a clear research need for simplified edge-implemented anomaly detection algorithms that train strictly on healthy operational data and flag deviations asynchronously.

- References

- Tang, Q., Shu, X., Zhu, G., Wang, J., & Yang, H. (2021). Reliability Study of BEV Powertrain System and Its Components—A Case Study. Processes, 9(5), 762. https://doi.org/10.3390/pr9050762

- Mahale, Y., Kolhar, S. & More, A.S. A comprehensive review on artificial intelligence driven predictive maintenance in vehicles: technologies, challenges and future research directions. Discov Appl Sci 7, 243 (2025). https://doi.org/10.1007/s42452-025-06681-3

- Shah, R., Mittal, V., & Lotwin, M. (2025). Recent Advances in Vibration Analysis for Predictive Maintenance of Modern Automotive Powertrains. Vibration, 8(4), 68. https://doi.org/10.3390/vibration8040068

- Yildirim, M., Polat, M., & Kürüm, H. (2014, September). A survey on comparison of electric motor types and drives used for electric vehicles. In 2014 16th International Power Electronics and Motion Control Conference and Exposition (pp. 218-223). IEEE.

- https://doi.org/10.1109/EPEPEMC.2014.6980715

- 1.Göndelen B, Karakaya A. Comparative Analysis of Motors Used in Electric Vehicles. JNSE. 2026;(Advanced Online Publication):95-122. https://doi.org/10.56850/jnse.1830251

- Cao, W., Bukhari, A. A. S., & Aarniovuori, L. (2019). Review of electrical motor drives for electric vehicle applications. Mehran University Research Journal of Engineering & Technology, 38(3), 525-540. https://doi.org/10.22581/muet1982.1903.01

- Ghosh, A., & Chatterjee, S. (2023). An overview on various sources of vibration in electric vehicle and their identification techniques. Journal of the Brazilian Society of Mechanical Sciences and Engineering, 45(8), 401. https://doi.org/10.1007/s40430-023-04318-3

- Ma, J., Xue, Y., Han, Q., Li, X., & Yu, C. (2022). Motor Bearing Damage Induced by Bearing Current: A Review. Machines, 10(12), 1167. https://doi.org/10.3390/machines10121167

- Huang, W., Du, B., Li, T., Cheng, Y., & Cui, S. (2025). Detection of PMSM demagnetization fault and eccentricity fault based on acoustic images and DeiT classifier. IEEE Transactions on Energy Conversion. https://doi.org/10.1109/TEC.2025.3550544.

- Tsoukas, V., Gkogkidis, A., Boumpa, E., & Kakarountas, A. (2024). A Review on the emerging technology of TinyML. ACM Computing Surveys, 56(10), 1-37.https://doi.org/10.1145/3661820

- Abduganiyeva, N., Samigov, B., Kosimova, M., Yusupova, R. N., Inagamova, G., & Musaev, U. (2025, October). Deep Learning-Driven Acoustic Diagnostics for EV Powertrain Maintenance Education. In 2025 Second International Conference on Intelligent Technologies for Sustainable Electric and Communications Systems (iTech SECOM) (pp. 1-6). IEEE. https://doi.org/10.1109/iTechSECOM64750.2025.11307554.

- Almaini, A., Folz, J., & Ashour, G. (2025, October). TinyML for Acoustic Anomaly Detection in IoT Sensor Networks. In 2025 5th International Conference on Electrical, Computer, Communications and Mechatronics Engineering (ICECCME) (pp. 1-5). IEEE. https://doi.org/10.1109/ICECCME64568.2025.11277514.

- Hyundai Motor Company & Kia Corporation, "AI-based device and method for diagnosing noise generation of electric vehicle PE parts," Korean Patent Application KR1020230121249A, Aug. 18, 2023.

- General Motors LLC, "Real-time damage detection and localization of damage in vehicle components using acoustic emission and machine learning," U.S. Patent US20190257794A1, Aug. 22, 2019.

Submitted by :

- Harish

- 2nd year ECE

- VIT Chennai